MSC CYBER SECURITY & PEN TESTING

CST4530

SECURITY SOLUTIONS & APPLICATIONS

LAB C2 NETWORK ACCESS CONTROL

JOHN ODI ETTA

M00783479

CST 4530 lab5 wk 9

TASK 1

A 2811 router is added and renamed R3 to the existing network. Similarly, Server 1 is directly connected to R3 using the FastEthernet  0/0 interface.

A WIC-1T WAN module was added to R3. Another WAN module was also added to R2. And configured using 192.168.0.5/30 on R2 and 192.168.0.6/30 on R3 respectively. Server 1 was also assigned the IP address 192.168.3.2, sub netmask 255.255.255.0, and gateway 192.168.3.1.

CONNECTIVITY TEST.

Ping server 1 from pc 1.

The ping request from pc1 to server 1 is successful.

Traceroute pc1 to server 1.

The network route is shown between pc 1 and server 1.

TASK2 Standard Access Control Lists

The IP address of pc1 was prevented from accessing R3 using the standard access-list 1.

Access-list was activated using the “IP access-group 1 out” command.

Ping from Pc1 to Server 1: Using standard ACL

Ping status from pc 1 to server failed due to ACL in place restricting access from Pc1 to Server.

Ping from R1 to Server1. The ping should succeed, if not, check your configurations.

Explain why the ping successfully completed?

R1 pinging server 1

The ping from R1 to Server 1 is successful because a standard ACL was implemented only on pc 1, denying only PC1, and permitting all other connections on the network.

Task 3: Extended Access Control Lists

Because of the ACLs activated, only pc 2 is allowed access to server 1. All other communication to sever 1 is blocked by the implicit denial of communication imposed by ACL.

Only Server1should be allowed to telnet R3 using Named ACL. Record your steps.

R3(config)#ip access-list extended notelnet

R3(config-ext-nacl)#exit

R3(config)#line?

line

R3(config)#line vty?

vty

R3(config)#line vty 0 4

R3(config-line)# access-class telnet in

R3(config-line)#^Z

R3#

Telnet R3 from Pc2

Telnet connection from Pc2 to R3 timed out.

Step 3: After step 2 of task 4 try to telnet R3 from PC2. Record your results

Pc 2 is unable to telnet R3.